### Re: Using relays to switch Channels (Questions)

by flemingmras » Sat Nov 28, 2009 2:08 pm

Here's your circuit revised and shows how the switching should be done.

There's just that fine line between stupid and clever - Nigel Tufnel

by joey » Sat Nov 28, 2009 3:30 pm

flemingmras wrote:Here's your circuit revised and shows how the switching should be done.

I would consider placing a DC reference to ground from the pole of the first relay

joey

There ya happy?

In this example the 10M resistors are added across the relay to ensure that the output coupling cap of the first stage is never "floating" and always maintains its DC reference during switching. This minimizes "popping" sounds as you switch channels.

Probably overkill but this is my preferred method to minimize gain loss from the first stage. It uses the B pole to ground the output of the unused signal path and reconfigure the voltage divider at the input of stage 3.

Perfect! Thank you for clearning that up!